# Interior Mixed-Light Worksheet

## Light Sources in the Room

Walk the room and log every source and its color before you set white balance. The goal is to reduce the number of light colors and decide a dominant one.

Window / daylight present and approximate color temperature: ________________________________

Tungsten / incandescent fixtures (which and how warm): ________________________________

Fluorescent sources and any green tint: ________________________________

LED sources and whether high or low CRI: ________________________________

## Reconciliation Plan

Decide how you will make the whites read white, then how any added light will match the room.

Dominant light chosen for white balance: ________________________________

Mismatched lamps to switch off or re-bulb: ________________________________

Gel for added flash/LED (CTO to warm to tungsten, CTB to cool, etc.): ________________________________

Gray card / known-white reference frame taken (Y/N): ________________________________

## Added Light and Reminders

Confirm supplemental light is clean and one technique to focus on this shoot.

Flash/LED bounced or diffused, never bare (Y/N): ________________________________

Flash power kept low to support, not overpower, the ambient: ________________________________

Flash frame and ambient frame shot from the locked tripod (Y/N): ________________________________

One technique to focus on (verticals, bracketing, gelling, retouching): ________________________________